Aim: Analysis of LTI System Responses to Standard Inputs Using Python

IDE:

Analyzing Discrete-Time Systems Using Z-Transform

The Z-transform is used for analyzing discrete-time signals and systems. The Z-transform of a discrete-time signal x[n] is given by:

Xz=n=-∞∞x[n]z-n

where z is a complex variable, Xz represents the Z-transform of the signal.

Z-Transform Function

For an LTI system, the Z-transform function H(z) is defined as:

Hz=B(z)A(z)=b0+b1z-1+⋯+bmz-ma0+a1z-1+⋯+anz-n

where B(z) is the numerator polynomial, A(z) is the denominator polynomial.

Stability

A discrete-time system is stable if all poles of its Z-transfer function lie inside the unit circle in the Z-plane. To check stability:

Calculate the poles of H(z)

Check if the magnitude of each pole is less than 1.

Causality

A system is causal if its impulse response h[n] is zero n<0. This generally means that the numerator polynomial should not have terms that depend on future values.

Time Invariance

A system is time-invariant if a time shift in the input results in an equivalent time shift in the output. For LTI systems, if the system is defined properly, it is generally assumed to be time-invariant.

Example

Hz=z2+0.5z2-1.5z+0.5

Bode Plot Analysis

Stability:

- Check the gain and phase margins.

- Ensure that both margins are positive for stability.

Causality:

- Examine the magnitude and phase at low frequencies.

- Confirm that the system behaves as a causal system (magnitude starts lower, phase starts near 0 and decreases).

Time Invariance:

- If the system is LTI, it is inherently time-invariant.

- Analyse the impulse response (if available) to verify consistent responses to delayed inputs.

Python Implementation

import numpy as np

import matplotlib.pyplot as plt

from scipy.signal import TransferFunction, lti

def analyze_z_transfer_function(num, den):

# Create a Transfer Function object

system = TransferFunction(num, den)

# Get the poles and zeros

zeros = system.zeros

poles = system.poles

print("Zeros:", zeros)

print("Poles:", poles)

# Stability Analysis

stable = all(np.abs(pole) < 1 for pole in poles)

print("Stability:", "Stable" if stable else "Unstable")

# Causality Analysis

causal = all(num[i] == 0 for i in range(len(num) - 1) if num[i + 1] == 0)

print("Causality:", "Causal" if causal else "Non-Causal")

# Time Invariance Analysis

time_invariant = True  # For Z-transforms, generally time-invariant if system defined properly

print("Time Invariance:", "Time Invariant" if time_invariant else "Time Variant")

# Bode plot (magnitude and phase)

w, mag, phase = bode(system)

# Plot Bode plot

plt.figure(figsize=(12, 8))

plt.subplot(2, 1, 1)

plt.semilogx(w, mag)    # Bode magnitude plot

plt.title('Bode Magnitude Plot')

plt.xlabel('Frequency [rad/s]')

plt.ylabel('Magnitude [dB]')

plt.grid()

plt.subplot(2, 1, 2)

plt.semilogx(w, phase)   # Bode phase plot

plt.title('Bode Phase Plot')

plt.xlabel('Frequency [rad/s]')

plt.ylabel('Phase [degrees]')

plt.grid()

plt.tight_layout()

plt.show()

# Example: Analyzing a specific system H(z) = (z^2 + 0.5)/(z^2 - 1.5z + 0.5)

num = [1, 0.5]  # Numerator coefficients

den = [1, -1.5, 0.5]  # Denominator coefficients

analyze_z_transfer_function(num, den)

Transfer Function:

Hz=0.51-0.8z-1

Causality: This system is causal because the denominator has a non-negative exponent (i.e., all powers of z-1 are non-negative).

Stability: The system is stable if the poles (the roots of the denominator) lie inside the unit circle. Here, the pole is z=0.8, which is inside the unit circle, so the system is stable.

Time Invariance: The system is time-invariant because the coefficients do not depend on time.

Transfer function:

Hz=1-z-11-0.5z-1

Causality: This system is causal.

Stability: The pole at z=0.5 is inside the unit circle, making the system stable.

Time Invariance: The system is time-invariant

Post Lab Exercise:

- Write a Python function to compute the Z-transform of an unit step function. verify whether the system is stable or unstable.

- Implement this for the system Hz=0.5z-0.7z-0.9z-0.6z-0.4 and verify whether the system is stable or unstable.